# Gazebo and ArduPilot setup for RFSonde

Windows + WSL2 Ubuntu 26.04 + Gazebo Jetty + custom ArduPilot SITL

Tested end to end on October 1 2026: Ubuntu 26.04.1 LTS on WSL2, Gazebo Jetty (gz-sim 10.5), ardupilot_gazebo plugin, the RFSonde ArduPilot fork in ~/Github/my-ardupilot, and the norman_radars world (KCRI, KOUN and ATD at Max Westheimer Airport, Norman, OK).

## How the pieces fit together

Windows:  Ground station     <-- MAVLink, TCP 127.0.0.1:5760 --+

|

WSL2:     ArduPilot SITL (--no-mavproxy) ---------------------+

(optional MAVProxy console on TCP 5762)

(accuracy-squares helper on TCP 5763, started by rfsonde_sitl.sh)

^

|  JSON physics link, UDP 9002 (stays inside WSL)

v

WSL2:     Gazebo + ArduPilotPlugin + norman_radars world (window shown by WSLg)

Gazebo and SITL run in the same WSL distribution. Only telemetry crosses over to Windows.

# 1  Prepare WSL and the GPU

In Windows PowerShell, update WSL and restart it (save your work in WSL first):

wsl --update

wsl --shutdown

In the Ubuntu terminal, check that OpenGL can use the NVIDIA GPU:

sudo apt-get update

sudo apt-get install -y mesa-utils

GALLIUM_DRIVER=d3d12 MESA_D3D12_DEFAULT_ADAPTER_NAME=NVIDIA glxinfo -B | grep renderer

Pass: the renderer reads D3D12 (NVIDIA ...). Without these two variables WSL falls back to llvmpipe (CPU software rendering) and Gazebo runs slowly. Step 3 sets them permanently.

# 2  Install Gazebo Jetty

Add the official Open Robotics package repository (it has Jetty for Ubuntu 26.04) and install Gazebo:

sudo apt-get install -y curl lsb-release gnupg

sudo curl -fsSL https://packages.osrfoundation.org/gazebo.gpg \

-o /usr/share/keyrings/pkgs-osrf-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) \

signed-by=/usr/share/keyrings/pkgs-osrf-archive-keyring.gpg] \

https://packages.osrfoundation.org/gazebo/ubuntu-stable $(lsb_release -cs) main" \

| sudo tee /etc/apt/sources.list.d/gazebo-stable.list > /dev/null

sudo apt-get update

sudo apt-get install -y gz-jetty

gz sim -v4 -r shapes.sdf

Pass: a Gazebo window opens with the shapes world. Close it. Never run gz with sudo.

# 3  Build the ArduPilot Gazebo plugin

sudo apt-get install -y build-essential cmake git rapidjson-dev libopencv-dev \

libgstreamer1.0-dev libgstreamer-plugins-base1.0-dev \

gstreamer1.0-plugins-bad gstreamer1.0-libav gstreamer1.0-gl

mkdir -p ~/Github && cd ~/Github

git clone https://github.com/ArduPilot/ardupilot_gazebo.git

cd ardupilot_gazebo

export GZ_VERSION=jetty

cmake -S . -B build -DCMAKE_BUILD_TYPE=RelWithDebInfo

cmake --build build -j4

## Set the environment (once)

Create env.sh and load it from ~/.bashrc so every new terminal finds the plugin, the RFSonde world, the ArduPilot models and the GPU:

cat > ~/Github/ardupilot_gazebo/env.sh <<'EOF'

export GZ_VERSION=jetty

export GZ_SIM_SYSTEM_PLUGIN_PATH=$HOME/Github/ardupilot_gazebo/build:$GZ_SIM_SYSTEM_PLUGIN_PATH

export GZ_SIM_RESOURCE_PATH=$HOME/Github/my-ardupilot/RFSonde-sim/models:\

$HOME/Github/my-ardupilot/RFSonde-sim/worlds:\

$HOME/Github/ardupilot_gazebo/models:$HOME/Github/ardupilot_gazebo/worlds:$GZ_SIM_RESOURCE_PATH

export GALLIUM_DRIVER=d3d12

export MESA_D3D12_DEFAULT_ADAPTER_NAME=NVIDIA

EOF

echo 'source "$HOME/Github/ardupilot_gazebo/env.sh"' >> ~/.bashrc

source ~/.bashrc

Terminals that were already open do not see changes to env.sh; open a new terminal or run source ~/Github/ardupilot_gazebo/env.sh. If the plugin is ever rebuilt for a different Gazebo version, delete build and rebuild from scratch.

# 4  Install the Norman radar-site world

The norman_radars world reproduces a 2 km radius around KCRI at Max Westheimer Airport: generic WSR-88D models for KCRI and KOUN, the ATD and its calibration tower at their surveyed positions, and the real OpenStreetMap layout of buildings, runways, roads and trees. It lives in the RFSonde ArduPilot repo, in ~/Github/my-ardupilot/RFSonde-sim:

~/Github/my-ardupilot/RFSonde-sim/

worlds/norman_radars.sdf      the world (generated)

models/norman_*               buildings, ground and tree meshes (generated)

tools/make_norman_world.py    regenerates the world (edit this, not the .sdf)

osm/norman_osm.json           OpenStreetMap data used by the script

README.md                     positions, sources and assumptions

The world and meshes are generated files and are not committed to git. Build them once after cloning (it takes under a second), and again whenever you edit the script:

python3 ~/Github/my-ardupilot/RFSonde-sim/tools/make_norman_world.py

Then register the takeoff point with SITL and have MAVProxy load its gimbal module at every start (run once):

mkdir -p ~/.config/ardupilot

echo "RFSonde_Norman=35.2380802,-97.4631246,358.6,135" >> ~/.config/ardupilot/locations.txt

echo "module load gimbal" >> ~/.mavinit.scr

~/.mavinit.scr runs every time MAVProxy starts, so the gimbal commands in step 6 are always available. The RFSonde_Norman location must match the world origin, or SITL GPS positions will be wrong. The origin is the drone takeoff point in an open field 200 m north and 50 m east of the ATD, and the drone starts facing south-east (heading 135°). Gazebo coordinates are metres East (x), North (y) and Up (z) from that point:

| Model | East (m) | North (m) | Height | RF reference frame |
| --- | --- | --- | --- | --- |
| KCRI | 260.6 | 23.0 | 30 m tower, antenna 34.7 m AGL | KCRI::antenna_center |
| KOUN | 70.7 | -224.6 | 20 m tower, antenna 24.7 m AGL | KOUN::antenna_center |
| ATD | -51.8 | -199.7 | radome centre 10.07 m AGL | ATD::radome_center, ATD::array_center |
| ATD_cal_tower | -39.8 | 225.3 | probe 44.63 m AGL | ATD_cal_tower::probe |

All radomes are 11.89 m (39 ft); the ATD radome size is not published and is assumed equal to the WSR-88D. Ground is 358.6 m MSL. Radars, building heights and the trees near the radar compound are generic; see RFSonde-sim/README.md for every source and assumption.

# 5  Fly SITL in the Norman world

Use two Ubuntu terminals. Always start Gazebo first.

# Terminal A - Gazebo

gz sim -v4 -r norman_radars.sdf

# Terminal B - your ArduPilot firmware

~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh

rfsonde_sitl.sh runs sim_vehicle.py with the RFSonde start location, the gimbal and standard-GPS parameters and --no-mavproxy, and shows the GPS accuracy squares (step 7). Add --gps sbas or --rtk to start with another GPS profile, or --no-squares to start without the squares; any other sim_vehicle.py option is passed on. Ctrl+C stops SITL and the squares.

The first run compiles the firmware, and the first world load takes a few seconds while the meshes are read. SITL opens its own window and waits until a ground station connects to TCP 5760. Only then does it start and link to Gazebo; its window prints JSON received.

## Connect the ground station

Connect your ground station to TCP 127.0.0.1:5760 (in Mission Planner: TCP, Connect, host 127.0.0.1, port 5760). Windows reaches the WSL port through localhost forwarding. When the ground station shows the drone at Max Westheimer Airport, SITL and Gazebo are linked.

## Optional MAVProxy console

The MAVProxy commands in this guide (test flight, gimbal) need a MAVProxy console. SITL also accepts a second connection on TCP 5762, so open one alongside the ground station, in Terminal C:

mavproxy.py --master tcp:127.0.0.1:5762 --console

It loads the gimbal module automatically (step 4). Do not add --out options to port 14550 if your ground station also listens there: every packet would arrive twice, and mission uploads fail with MAV_MISSION_INVALID_SEQUENCE.

## Test flight

From the ground station (GUIDED mode, arm, take off to 10 m), or in the MAVProxy console (Terminal C):

mode guided

arm throttle

takeoff 10

Pass: the Iris climbs to 10 m with KCRI, KOUN and the ATD visible around it, and Mission Planner shows it at Max Westheimer Airport. Arming is refused for about 35 s after start-up while the EKF settles; just retry. Stop with Ctrl+C in each terminal.

SITL turns Gazebo positions into lat/lon with a simplified Earth model, so its GPS reads about 0.2% of the distance from takeoff away from the true position (under 1 m at the radars). Gazebo geometry is exact, so take RF ranges from Gazebo.

# 6  Gimbal setup and usage

## How the gimbal is set up

The drone in the world is rfsonde_iris: the ArduPilot Iris carrying rfsonde_gimbal, a 3-axis servo gimbal (joint order vehicle → yaw → roll → pitch). Both are RFSonde copies of the ardupilot_gazebo models, kept in RFSonde-sim/models so the upstream plugin stays untouched. ArduPilot drives the gimbal as a servo mount (MNT1_TYPE 1) through SERVO9–11; the Gazebo plugin turns those PWM outputs into joint angles. The parameters come from RFSonde-sim/config/rfsonde-gimbal.parm, which step 5 loads.

| Axis | ArduPilot limits | Positive direction | Servo / RC | Gazebo joint |
| --- | --- | --- | --- | --- |
| Roll | -30 to +30° | right side down | SERVO9 / RC6 | roll_joint (same sign) |
| Pitch | -135 to +45° | up | SERVO10 / RC7 | pitch_joint (opposite sign) |
| Yaw | -160 to +160° | right (clockwise) | SERVO11 / RC8 | yaw_joint (opposite sign) |

The gimbal starts in MAVLink targeting mode at neutral: level and pointing along the drone's nose.

## Read the frame triad

A reference-frame triad is drawn on the antenna frame antenna_rf and moves with the gimbal:

X (red): the boresight, where the antenna points. It continues as a thin red 300 m laser line and a translucent yellow beam cone (30° beamwidth, 20 m long).

Y (green): points left at neutral.

Z (blue): points up at neutral. This is the vertical-polarization reference, so roll shows as Y and Z turning around the red boresight.

Pitch tilts X and Z, and yaw swings X and Y. The triad, laser and cone have no mass or collision and are hidden from the gimbal camera image. To change the cone, edit pointer_beam_cone in rfsonde_gimbal/model.sdf (radius = length × tan(beamwidth / 2)). Delete the pointer_* visuals to hide everything.

## Point the gimbal at angles

In the MAVProxy console (Terminal C, step 5), give roll, pitch and yaw in degrees relative to the drone. The gimbal module loads automatically (step 4); without that setup, run module load gimbal first.

gimbal point 0 -30 40

gimbal point 0 0 0

The first command aims 30° down and 40° right; the second returns to neutral. To fly the gimbal with RC inputs instead, switch modes and set PWM 1100–1900 on RC6–8:

gimbal mode RC

rc 6 1100

rc 6 1500

RC 1500 is the middle of each range, which for pitch is -45°, not level. gimbal point switches back to MAVLink targeting.

## Point the gimbal at a radar

Set a region of interest (ROI) and ArduPilot keeps the gimbal aimed at it while the drone moves. The altitude is metres above the takeoff point:

param set MNT1_ARRC_GMODE 0

long DO_SET_ROI_LOCATION 0 0 0 0 35.2382871 -97.4602619 34.69

| Target | Latitude | Longitude | Height above takeoff | Bearing from takeoff |
| --- | --- | --- | --- | --- |
| KCRI antenna | 35.2382871 | -97.4602619 | 34.69 m | 85.0° |
| KOUN antenna | 35.2360553 | -97.4623481 | 24.69 m | 162.5° |
| ATD radome centre | 35.2362802 | -97.4636935 | 10.07 m | 194.5° |
| ATD calibration probe | 35.2401106 | -97.4635615 | 44.63 m | 350.0° |

Yaw limit: the gimbal turns only ±160° from the nose, and ArduCopter does not turn the drone when the gimbal has its own yaw. For a target behind the drone, turn the drone first with setyaw BEARING 30 0 (for example setyaw 194.5 30 0 for the ATD), then set the ROI.

RFSonde ROI modes: MNT1_ARRC_GMODE selects how your firmware aims at a ROI. 0 is standard ArduPilot pointing. 1 keeps the probe plane parallel to the antenna under test, 2 aligns for H-pol and 3 for V-pol, using MNT1_ARRC_AZTH (antenna-under-test azimuth), MNT1_ARRC_ELEV and MNT1_ARRC_ZOFF. The SITL defaults set mode 2, so set 0 for plain aiming.

Pass: with the ROI on KCRI, the red laser runs into the KCRI radome and the gimbal camera shows KCRI in the centre of its image.

## Read the actual gimbal angles

Gazebo publishes the achieved joint angles (radians), which is what RF calculations should use:

gz topic -e -t /world/norman_radars/model/rfsonde_iris/model/gimbal/joint_state

Pitch and yaw joints have the opposite sign to ArduPilot's angles (see the table above). For full 3D pointing, use the world pose of antenna_rf rather than combining angles by hand.

Your firmware needs the AP_Mount_Servo fix from October 2026 for the gimbal to hold its angles in flight. The RFSonde mount parameters reuse the storage of the stock servo lead-filter gains, and without the fix roll and pitch slam to their limits as soon as the drone rotates.

# 7  GPS accuracy, RTK and accuracy squares

## How the simulated GPS works

SITL simulates the GPS receiver; Gazebo only supplies the true position. The RFSonde GNSS error model adds, on each axis, a slowly varying error (as from the atmosphere and multipath) plus fast white noise, and noise on the velocity. The simulated receiver reports its 1-sigma accuracy from these settings, and RTK Fixed when the horizontal accuracy is below 10 cm. The statistics do not depend on the GPS update rate. The model is in libraries/SITL/SIM_GPS.h (GNSSErrorModel).

| Parameter | Meaning |
| --- | --- |
| SIM_GPS_HBIAS / SIM_GPS_VBIAS | Slowly varying error, 1-sigma per horizontal axis / vertical (m) |
| SIM_GPS_BTAU | Correlation time of that error (s): how long it takes to change |
| SIM_GPS_HWN / SIM_GPS_VWN | White noise on every sample, horizontal per axis / vertical (m) |
| SIM_GPS_VELNSE | Velocity noise per axis (m/s) |
| SIM_GPS_SEED | Random seed; 0 = new each start, any other value repeats the same error |

Three profiles in RFSonde-sim/config set these for a u-blox ZED-F9P class receiver, together with matching EKF3 GPS noise settings:

| Profile | Horizontal (CEP50) | Vertical (1-sigma) | Ground station shows | EKF3 GPS noise floor |
| --- | --- | --- | --- | --- |
| standard (gps-standard.parm) | about 1.5 m | about 1.9 m | 3D Fix | EK3_POSNE_M_NSE 0.5 (default) |
| sbas (gps-sbas.parm) | about 1.0 m | about 1.3 m | 3D Fix | EK3_POSNE_M_NSE 0.5 (default) |
| rtk (gps-rtk.parm) | about 1.1 cm | about 1.4 cm | RTK Fixed | EK3_POSNE_M_NSE 0.05, EK3_VELNE_M_NSE 0.1 |

All profiles set EK3_SRC1_POSZ 3, so ArduPilot uses GPS height instead of the barometer.

EKF3 weighs GPS position by the larger of the reported accuracy and EK3_POSNE_M_NSE. The RTK profile lowers that floor; with the default 0.5 m, EKF3 would treat RTK as half-metre data.

EKF3 smooths the fast noise, but it cannot remove the slowly varying error: nothing else gives it an absolute position. Its estimate, and the drone, follow that error, as on the real aircraft.

## Switch GPS profiles

Start with a profile: rfsonde_sitl.sh (standard), rfsonde_sitl.sh --gps sbas or rfsonde_sitl.sh --rtk. Switch at any time, also in flight, in the MAVProxy console (Terminal C):

param load ~/Github/my-ardupilot/RFSonde-sim/config/gps-rtk.parm

param load ~/Github/my-ardupilot/RFSonde-sim/config/gps-sbas.parm

param load ~/Github/my-ardupilot/RFSonde-sim/config/gps-standard.parm

In Mission Planner use Config → Full Parameter List → Load from file, choose the file, then Write Params.

Pass: the ground station's GPS status changes between 3D Fix and RTK Fixed within a second.

The error switches to the new size at once (no RTK float stage), so a hovering drone moves by up to the previous error.

Home altitude is set when the drone arms. If you arm in standard mode and then enable RTK, a constant height offset (the GPS height error at arming) remains until you land and arm again.

## Evaluate GNSS and EKF accuracy

gnss_flight_test.py flies a repeatable test (takeoff, hover, box pattern, landing) and evaluates the log; gnss_eval.py evaluates any SITL log. With SITL running (step 5), in another terminal:

python3 ~/Github/my-ardupilot/RFSonde-sim/tools/gnss_flight_test.py --profile rtk

python3 ~/Github/my-ardupilot/RFSonde-sim/tools/gnss_flight_test.py --profile standard --hover 300 --box 30

python3 ~/Github/my-ardupilot/RFSonde-sim/tools/gnss_eval.py --latest

The flight test connects on TCP 5762, so close the MAVProxy console first (or pass --mavlink with another free SITL port); your ground station on 5760 can stay connected. The report compares the raw GPS and the EKF3 estimate with the true position:

| Report section | What it tells you |
| --- | --- |
| Whole flight / Hovering / Moving | Error per axis (1-sigma and mean), horizontal RMS, CEP50, CEP95 and vertical RMS, for GPS and EKF3 |
| Fast noise RMS | The noise faster than about 2 s: EKF3 should show less than the GPS |
| EKF follows the GPS error | Correlation between EKF3 and GPS errors: close to 1 means EKF3 tracks the slowly varying GPS error |
| Reported vs actual accuracy | Whether the receiver's accuracy reports match the real error |
| Correlation time | How quickly the GPS error changes (compare with SIM_GPS_BTAU) |
| Timing | Measured GPS delay, and how far the EKF3 output trails the truth (about 20–30 ms in SITL, which adds about speed × lag to the error while moving) |
| EKF3 innovation test ratios | 0 = GPS agrees with the EKF3 prediction, 1 = GPS rejected; values well below 1 mean the EKF3 noise settings fit the GPS |

The report is saved next to the log as *_gnss_report.txt; --csv FILE also exports the error time series. With a correlation time of a minute, a single short flight can read 20% above or below the configured accuracy; set SIM_GPS_SEED to compare configurations on the same error sequence. To check the error model's own statistics, record a long run (several times SIM_GPS_BTAU, 20 minutes or more for the standard profile), for example on the ground with LOG_DISARMED 1 and SIM_SPEEDUP 10, then run gnss_eval.py --latest --all.

## Accuracy squares

The accuracy squares show the position uncertainty around the drone in the Gazebo window:

Red square, 1 m sides, with a matching vertical square.

Orange square, 60 cm sides, with a matching vertical square.

They are centred on where ArduPilot believes the drone is, level with the ground and turned with the drone's heading; the vertical squares stand across the drone (left-right and up). The drone model is always at its true position, so it moves inside the squares by the actual position error: about a metre in standard mode, a few centimetres with RTK.

The squares are on by default: rfsonde_sitl.sh starts a helper script (accuracy_squares.py) that reads ArduPilot's position on TCP 5763 and draws them. They appear once the ground station has connected and SITL is running.

## Enable or disable the squares

| To | Do this (any terminal) |
| --- | --- |
| Hide the squares | python3 ~/Github/my-ardupilot/RFSonde-sim/tools/accuracy_squares.py --off |
| Show them again | python3 ~/Github/my-ardupilot/RFSonde-sim/tools/accuracy_squares.py --on |
| Show only the red 1 m square | add --on --show large |
| Show only the orange 60 cm square | add --on --show small |
| Start SITL without squares | ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh --no-squares |
| Draw squares for a SITL started with sim_vehicle.py directly | python3 ~/Github/my-ardupilot/RFSonde-sim/tools/accuracy_squares.py (Ctrl+C removes them) |

--on and --off act on the running helper. The squares are Gazebo markers: they appear in the Gazebo window only, have no mass or collision, and do not show in the gimbal camera image. They follow the drone at 10 updates per second. The helper's log is ~/.cache/rfsonde/accuracy_squares.log.

# 8  Add the RFSonde antenna and sensors

## Mount the antenna

In rfsonde_gimbal/model.sdf, replace the camera visual on pitch_link with your antenna geometry and set realistic mass and inertia for each moving link.

Keep the SDF joint limits and the ArduPilot MNT1_* limits the same as the real gimbal.

Set the antenna_rf frame pose in rfsonde_gimbal/model.sdf to your antenna phase centre. It is relative to pitch_link; keep +X on the boresight and +Z on the vertical polarization. The triad, laser and cone follow it.

## Add a sensor (example: IMU on the antenna)

Put the sensor inside the antenna <link>. The norman_radars world already loads the IMU and sensor systems.

<sensor name="antenna_imu" type="imu">

<always_on>true</always_on>

<update_rate>100</update_rate>

<topic>/rfsonde/antenna_imu</topic>

</sensor>

gz topic -l

gz topic -e -t /rfsonde/antenna_imu

A Gazebo sensor only publishes to a Gazebo topic; it does not feed ArduPilot. RFSonde code reads it with Gazebo Transport (Python or C++).

## RF measurements (next step)

Gazebo supplies geometry only: antenna pose, gimbal angles, range and line of sight. RF power and phase come from your own model. Buildings and trees have no RF properties.

Take radar positions from the frames in step 4 (for example ATD::array_center) and the drone antenna from antenna_rf. Range and direction come from their world poses.

Start with a free-space link budget: Pr = Pt + Gt + Gr − 20·log10(4πR/λ) − losses (dB), using the antenna pattern at the transmitter direction in the antenna_rf frame. Gimbal roll adds polarization loss.

Prototype as an external script that subscribes to pose topics; move it to a C++ Gazebo system plugin later if you need sample-accurate timing.

# 9  Troubleshooting

"Unable to find or download file norman_radars.sdf" (or Fuel fetch failed): the terminal was opened before env.sh was updated or loaded. Run source ~/Github/ardupilot_gazebo/env.sh, open a new terminal, or launch it by full path: gz sim -v4 -r ~/Github/my-ardupilot/RFSonde-sim/worlds/norman_radars.sdf. If that file does not exist, generate it (step 4).

SITL dies right after Gazebo connects (for example *** buffer overflow detected ***, or arducopter just disappears): the crash is in the ArduPilot firmware, not Gazebo. Confirm with sudo dmesg | grep arducopter, then add --gdb to the sim_vehicle.py command to get a backtrace. On October 1 2026 this was a buffer overflow in AP_ARRC_LB5900::configSensor(), now fixed.

Mission Planner shows the drone in Australia or the wrong place: -L RFSonde_Norman is missing, or the entry in ~/.config/ardupilot/locations.txt does not match the world origin.

Gimbal points backwards or sideways at start: SITL was started without rfsonde-gimbal.parm, so the gimbal follows RC6–8 defaults. Use the step 5 command, or run gimbal point 0 0 0.

Gimbal roll or pitch jumps to its limits in flight: the firmware is missing the AP_Mount_Servo fix (step 6). Rebuild SITL from the fixed branch.

"Another world of the same name is running": an old Gazebo is still open. Run pkill -f gz-sim and start again.

SITL waits forever / no `JSON received`: with --no-mavproxy, SITL does not start until a ground station connects to TCP 5760. Also start Gazebo first, with -r, or press Play in the Gazebo window. Only one Gazebo vehicle can use UDP 9002 at a time.

Ground station cannot connect to 127.0.0.1:5760: check that SITL is running (ss -ltn | grep 5760 in WSL) and that only one SITL instance is running. A ground station on Windows needs WSL localhost forwarding, which is on by default.

Accuracy squares do not appear: start SITL with rfsonde_sitl.sh (not sim_vehicle.py directly), connect the ground station, and check that they were not switched off (accuracy_squares.py --on). The log ~/.cache/rfsonde/accuracy_squares.log should end with connected to SITL and accuracy squares: large,small.

Ground station still shows 3D Fix after loading gps-rtk.parm: check that the parameters were written (SIM_GPS_HBIAS 0.008, SIM_GPS_HWN 0.005). The firmware needs the RFSonde GNSS error model (SIM_GPS.h, October 2026); rebuild SITL.

Mission upload fails with MAV_MISSION_INVALID_SEQUENCE, or every packet arrives twice: two links deliver the same stream to the ground station, usually a MAVProxy --out to the port it listens on. Remove the extra output.

Gazebo is slow, or `glxinfo -B` shows llvmpipe: the GPU variables are missing (step 3). If the Gazebo window misbehaves on the GPU, test with unset GALLIUM_DRIVER MESA_D3D12_DEFAULT_ADAPTER_NAME. Gazebo and SITL run in lockstep, so a real-time factor below 1 slows the clock but keeps the flight dynamics correct.

Plugin fails to load after a Gazebo upgrade: delete ~/Github/ardupilot_gazebo/build and repeat step 3.

# References

Gazebo Jetty installation on Ubuntu

ArduPilot Gazebo plugin README

ArduPilot: SITL with Gazebo

ArduPilot: JSON simulation interface

MAVProxy telemetry forwarding

ArduPilot gimbal parameter file for Gazebo

ArduPilot gimbal (mount) targeting modes

Gazebo sensors tutorial

Gazebo system plugins

Gazebo Jetty troubleshooting

Microsoft: WSL GUI apps (WSLg)

Microsoft: WSL networking (NAT and mirrored)

Norman world data: radar positions from the NWS ROC test-bed notice, the NEXRAD site surveys for KOUN (1987) and OSF-3/KCRI (1994), and the RFSonde ATD survey. Buildings, runways, roads and trees © OpenStreetMap contributors (ODbL).