RFSonde Cheatsheet

Gazebo SITL in the Norman radar world, the RFSonde companion computer (LattePanda) and the ground station. Autopilot branch ArduPilot-4.7-ARRC. Commands run in WSL unless marked Windows or LattePanda. Details: RFSonde-sim/README.md, RFSonde_Computer/docs/OPERATIONS.md.

1. One-time setup (WSL)

| cd ~/Github/my-ardupilot && git checkout ArduPilot-4.7-ARRC | RFSonde autopilot branch |
| --- | --- |
| git submodule update --init --recursive | fetch submodules (mavlink comes from the my-mavlink fork); repeat after a pull that moves them |
| source ~/venv-ardupilot/bin/activate | Python env with MAVProxy and pymavlink (if not already active) |
| cd ~/Github/ardupilot_gazebo/build cmake .. -DCMAKE_BUILD_TYPE=RelWithDebInfo && make -j4 | build the ArduPilot Gazebo plugin |
| source ~/Github/ardupilot_gazebo/env.sh | Gazebo Jetty paths, RFSonde models and GPU rendering (already in ~/.bashrc) |
| grep RFSonde_Norman ~/.config/ardupilot/locations.txt | SITL start location; must read 35.2380802,-97.4631246,358.6,135 |
| echo "module load gimbal" >> ~/.mavinit.scr | MAVProxy loads its gimbal module at every start |

2. Norman radar world (Gazebo)

| python3 ~/Github/my-ardupilot/RFSonde-sim/tools/make_norman_world.py | generate the world and meshes from OSM (once after cloning, and after editing the script) |
| --- | --- |
| gz sim -v4 -r norman_radars.sdf | Terminal A: Gazebo with its window, running |
| gz sim -s -r --headless-rendering norman_radars.sdf | Gazebo server only, no window |
| gz sim -g | open the Gazebo window for a running server |
| gz topic -l | list Gazebo topics |
| gz topic -e -t /world/norman_radars/model/rfsonde_iris/\ model/gimbal/joint_state | achieved gimbal joint angles (rad); one line, without the \ |

Start order: Gazebo (A), then SITL (B), then connect the ground station to TCP 127.0.0.1:5760. SITL does not start, or link to Gazebo, until that connection is made. Stop each terminal with Ctrl+C.

3. SITL (ArduCopter on the Gazebo Iris)

| ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh | Terminal B: SITL at RFSonde_Norman, gimbal and standard GPS params, accuracy squares, no MAVProxy |
| --- | --- |
| ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh --rtk | same, with the RTK GPS profile |
| ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh --gps sbas | same, with the SBAS GPS profile |
| ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh --no-squares | without the accuracy squares |
| ~/Github/my-ardupilot/RFSonde-sim/tools/rfsonde_sitl.sh -w | wipe stored parameters back to defaults (any sim_vehicle.py option is passed on) |
| mavproxy.py --master tcp:127.0.0.1:5762 --console --map | Terminal C (optional): MAVProxy console and map |
| cd ~/Github/my-ardupilot Tools/autotest/sim_vehicle.py -v ArduCopter -L RFSonde_Norman --no-mavproxy | plain SITL without Gazebo (quick tests) |

Don't give MAVProxy an --out to a port the ground station listens on: packets arrive twice and mission uploads fail. -I1 runs a second SITL with every port +10.

SITL ports (instance 0)

| Port | Use |
| --- | --- |
| TCP 5760 (SERIAL0) | ground station: Ludifly ("WSL SITL") or Mission Planner |
| TCP 5762 (SERIAL1) | MAVProxy console, gnss_flight_test.py, or the companion-computer simulator |
| TCP 5763 (SERIAL2) | accuracy-squares helper |
| UDP 9002 | Gazebo plugin <-> SITL physics (JSON); -I1 uses 9012 |

4. Accuracy squares (from RFSonde-sim)

| python3 tools/accuracy_squares.py --off | hide the 1 m and 60 cm squares |
| --- | --- |
| python3 tools/accuracy_squares.py --on | show them again |
| python3 tools/accuracy_squares.py --on --show large | 1 m square only (small = 60 cm only) |
| python3 tools/accuracy_squares.py | squares for a SITL started with sim_vehicle.py directly; Ctrl+C removes them |
| tail -f ~/.cache/rfsonde/accuracy_squares.log | helper log |

5. MAVProxy commands (Terminal C)

| mode guided arm throttle takeoff 20 | take off to 20 m |
| --- | --- |
| mode rtl / mode land | return home / land here |
| param load ~/Github/my-ardupilot/RFSonde-sim/config/gps-rtk.parm | switch GPS profile live (gps-standard.parm, gps-sbas.parm) |
| param set MNT1_ARRC_GMODE 0 | ROI aiming: 0 ArduPilot, 1 probe plane parallel to the AUT, 2 H-pol, 3 V-pol (SITL default 2) |
| gimbal point 0 -30 40 | gimbal roll, pitch, yaw (deg) |
| gimbal mode RC | gimbal on RC6/7/8 (rc 7 1500) |
| long DO_SET_ROI_LOCATION 0 0 0 0 35.2382871 -97.4602619 34.69 | aim at the KCRI antenna |
| setyaw 194.5 30 0 | turn the vehicle to face the ATD (gimbal yaw is limited to +/-160 deg) |
| wp load mission.txt / wp list | upload / show a waypoint mission |

6. Flight tests and logs (from RFSonde-sim)

| python3 tools/gnss_flight_test.py --profile rtk | automatic takeoff, hover, box, land, then GNSS/EKF3 report (uses TCP 5762) |
| --- | --- |
| python3 tools/gnss_flight_test.py --profile standard --hover 300 --box 30 | longer hover, 30 m box |
| python3 tools/gnss_eval.py --latest | GNSS and EKF3 accuracy report for the newest SITL log |
| ls -t ~/Github/my-ardupilot/logs ¦ head | newest SITL logs (.BIN) |
| MAVExplorer.py ~/Github/my-ardupilot/logs/00000012.BIN | plot a log |
| mavlogdump.py --types ARGM,MNT logs/00000012.BIN | print selected log messages |

7. Ground station and companion simulator

| cd C:\Users\sega0000\Github\RFSonde_GUI .\.venv\Scripts\Activate.ps1 ludifly | Windows PowerShell: run Ludifly from source; connect to "WSL SITL (TCP 5760)" |
| --- | --- |
| cd /mnt/c/Users/sega0000/Github/RFSonde_Computer pip install -e .[dev] | once: companion software from source (in a venv) |
| python3 -m rfsonde_cc -c config/rfsonde_cc_sitl.yaml \ --url tcp:127.0.0.1:5762 --data /tmp/rfsonde-data | simulated bladeRF fed by the Gazebo SITL pose, on SERIAL1 (skip the MAVProxy console) |

8. SSH to the RFSonde computer (LattePanda)

| netsh interface ipv4 set address name="Realtek-USB" static 192.168.77.10 255.255.255.0 | Windows, admin terminal: PC Ethernet on the direct-cable subnet |
| --- | --- |
| netsh interface ipv4 set address name="Realtek-USB" dhcp | Windows, admin: back to DHCP for normal networks |
| ping 192.168.77.1 | check the cable link |
| ssh rfsonde@192.168.77.1 | direct Ethernet cable (WSL or Windows) |
| ssh rfsonde@rfsonde-pc.local | same network, by name (Windows only: WSL cannot resolve .local) |
| ssh rfsonde@192.168.1.54 | lab network (DHCP address, can change) |
| ssh rfsonde@10.42.0.1 | the LattePanda's own Wi-Fi hotspot (starts 40 s after boot without a known network) |
| ssh-copy-id rfsonde@192.168.77.1 | once: log in with your key instead of the password |
| ssh -t rfsonde@192.168.77.1 rfsonde-ctl | status without opening a shell |

9. On the LattePanda

| rfsonde-ctl | status: release, service, autopilot link, SDR, recording, disk, network |
| --- | --- |
| rfsonde-ctl logs -f | follow the service log (-b this boot, --since "10 min ago") |
| rfsonde-ctl restart (stop / start) | the service; a stop lasts until start or the next boot |
| rfsonde-ctl check | full check with the bladeRF and the autopilot (stops the service; refused while armed) |
| rfsonde-ctl selftest | check the active release without stopping the service |
| rfsonde-ctl config (config show / config diff) | edit, show, or diff the configuration |
| rfsonde-ctl link / sudo rfsonde-ctl link auto | autopilot port and serial devices / pick the Cube |
| rfsonde-ctl versions / sudo rfsonde-ctl rollback | installed releases / back to the previous one |
| sudo rfsonde-ctl update --usb | install the newest release from a USB flash drive |
| sudo rfsonde-ctl export --today | copy today's recordings to a USB flash drive (--since 20261010) |
| rfsonde-ctl ethernet / rfsonde-ctl wifi | network state (wifi add SSID, wifi ap, wifi client) |
| ls /dev/serial/by-id/ ; lsusb | Cube (-if00 = MAVLink) and bladeRF on USB |
| df -h /data | free space for recordings |
| sudo reboot / sudo shutdown -h now | restart / power down cleanly |

10. Release and data (WSL)

| cd ~/RFSonde_Computer git pull /mnt/c/Users/sega0000/Github/RFSonde_Computer main | bring the build copy up to date |
| --- | --- |
| ./packaging/build.sh --mavlink-src ~/Github/my-mavlink | build the release tarball into dist/ |
| tools/deploy.sh rfsonde@192.168.77.1 | copy and install the newest release (asks for the sudo password) |
| rsync -avP rfsonde@192.168.77.1:/data/rfsonde/ ~/rfsonde-data/ | pull the HDF5 recordings (only new files on later runs) |
| scp -r rfsonde@rfsonde-pc.local:/data/rfsonde/20261014 . | Windows PowerShell: one day of recordings |

11. Autopilot firmware

| cd ~/Github/my-ardupilot ./waf configure --board CubeOrangePlus-ODID && ./waf copter | build for the vehicle's Cube Orange+; output build/CubeOrangePlus-ODID/bin/arducopter.apj |
| --- | --- |
| ./waf configure --board sitl && ./waf copter | build SITL by hand (sim_vehicle.py does it itself) |

Flash the .apj with Mission Planner, Install Firmware, Load custom firmware: WSL cannot see the Cube's USB port.